TL0 – TL1 Layer Adaptation

V1.0.2 – June 26, 2003

Yann Bajot, Prosilog

Stéphane Guntz, Prosilog

OCP International Partnership (OCP-IP) disclaims all warranties and liability for the use of this document and the information contained herein and assumes no responsibility for any errors that may appear in this document, nor does OCP-IP make a commitment to update the information contained herein.

Contact the OCP-IP office to obtain the latest revision of this document.

Questions regarding this document or membership in OCP-IP may be forwarded to:

OCP-IP

www.ocpip.org

E-mail: admin@ocpip.org

Phone: +1 503-291-2560

Fax: +1 503-297-1090

OCP-IP Technical Support

techsupport@ocpip.org

DISCLAIMER

This OCP-IP document is provided "as is" with no warranties whatsoever, including any warranty of merchantability, noninfringement, fitness for any particular purpose, or any warranty otherwise arising out of any proposal, specification or sample.  OCP-IP disclaims all liability for infringement of proprietary rights, relating to use of information in this document.  No license, express or implied, by estoppel or otherwise, to any intellectual property rights is granted herein.

All product names are trademarks, registered trademarks, or servicemarks of their respective owners.

Copyright (C) 2003 OCP-IP

# Introduction

Layer adapters presented here are intended to illustrate the TL0-TL1 layer adaptation process for the OCP-based transaction level communication interface.

These adapters are given as examples and implement only a subset of the whole OCP 1.0 dataflow interface. However, an effort was made here to address the main problems encountered during the layer adaptation process. The solutions proposed to solve these problems are explained and can be reused to extend the adapters to support more OCP features (see section 4). Note that we only consider the dataflow part of the OCP interface here (no sideband signals).

The model files referred here are stored in package "ocp_tl0_tl1_layer_adapters_examples.tar.gz", which should be unzipped in the "tl_sc" root directory.

TL0-TL1 adapters are essential to connect TL1 systems to the TL0 (RTL) world: synthesizable SystemC, or VHDL/Verilog. Two kind of adapters are necessary to connect TL1 systems to TL0 slaves or TL0 masters.

Main issues concern how to manage signal glitches and propagation delays, which are very common for TL0 models. Signals coming from TL0 parts are not supposed to be clocked and therefore can change late in the clock cycle. It is also possible that some glitches appear on them, especially in the case of co-simulation with a HDL-simulated RTL Slave.

Once an interface method call has been done to a TL1 channel, it cannot be canceled anymore. Hence, adapters must issue only one TL1 call per cycle, at the right time when TL0 signals are supposed to be stable. Note that co-simulation with HDL-simulated modules is not addressed here, and requires the use of simulator-specific APIs such as FLI and PLI.

IMPORTANT NOTE : All the adapters presented here use events of the communication class to enable synchronization. As a result, TL1 channels connected to the adapters port MUST have its 'syncevent' constructor parameter set (see the SystemC OCP Transaction Level Communication Channel documentation page 4).

# TL0 Slave Adapters

These adapters allow to connect a TL0 Slave to a TL1 system. In this section, we will present two examples of adapters that implement two different subsets of the OCP interface:

- Basic adapter: All the Basic signals (Clock, MAddr, MCmd, Mdata, ScmdAccept, Sdata, Sresp) + MRespAccept

- Basic adapter with datahandshake: All the Basic signals + MRespAccept + Datahandshake signals (MdataVaild + SDataAccept)

Four examples are provided (three for the basic adapter and one for the datahandshake adapter), which present the use of the slave adapters with different kinds of TL0 slaves and TL1 masters.

## Basic TL0 Slave adapter

The code for this adapter can be found in files layer_adapters/tl0_tl1/ocp_tl0_tl1_slave_adapter(.h,cpp). During an OCP transaction, the adapter should behave as follows:

- For the request phase, the adapter catches each sent request on the TL1 Slave Port, extracts the TL1 data fields and drives the corresponding TL0 Request group signals. Then, it waits for ‘SCmdAccept’ to be asserted by the slave and releases the request on TL1 channel.

- For the response phase, the adapter samples Response group signals driven by the TL0 Slave, test the ‘SResp’ signal to identify a valid response and conditionally put it on the TL1 channel. Then, it waits for the TL1 Master to release the response and asserts the ‘MrespAccept’ signal during one cycle.

Figure 1 TL0 Slave Adapter

The adapter has to deal with potential late changes and glitches on its TL0 input signals. For a TL0 Slave, it concerns the following signals:

- SCmdAccept

- SResp and SData signals

The slave adapter issues some TL1 calls that can potentially change the state of the channel:

- Srelease() calls to release the request sent by the TL1 system

- SputResponse() calls to send response to the TL1 system

Once one of these calls has be done, the state channel has changed and there is no way to return to the previous state. Hence, it is essential to be sure that the TL0 signal values driving the TL1 calls are the good ones, i.e. they will not change until the end of the cycle.

The 'SputResponse()' and associated data class calls must happen only when TL0 associated signals 'SResp' and 'SData' are stable. Since the propagation delays for these signals are slave dependent, a 'Response Sample Delay' parameter is used to determine the precise instant when these signals should be sampled during the cycle (cf. Figure 2). Once sampled, 'SResp' is tested; if the response is valid (DVA value), dataclass response and 'SputResponse()' calls are done.

In the same manner; the 'Srelease()' call must happen only when 'SCmdAccept' is stable. Since the propagation delay for this signal is slave dependent, a 'CmdAcceptSampleDelay' parameter is used to determine when 'SCmdAccept' should be sampled during the cycle (cf. Figure 2).

Figure 2 Slave Adapter sample delays

### Default constructor

This constructor give defaults values to sampling delays. These values are compatible with the OCP Level 2 timing guidelines.

OCP_TL0_TL1_Slave_Adapter(sc_module_name name,

, int ID

, bool Combinational

, sc_time ClockCycle

, bool Check_setup_time = true

)

- name: Name of the module (slave adapter) instance.

- ID: Unique number identifying the Slave.

- Combinational: Boolean specifying if there is a combinational path between Request and Response signals. Sampling times will be set as follows:

Combitional:

* RequestSampleDelay = ClockCycle*99/100

* ResponseSampleDelay = ClockCycle*60/100

- Sequential:

* RequestSampleDelay = ClockCycle*99/100

* ResponseSampleDelay = ClockCycle*60/100

- ClockCycle: sc_time number specifying the OCP clock period

- Check_setup_time: boolean value specifying if 'check_setup_time()’ function is executed during simulation (see below).

'Check_setup_time()' function is proposed for debugging purposes, to ensure that TL0 slave output signals do not change after the sample delays, which would cause the adapter to fail. A good way to proceed is to perform the first simulation with checking function activated to determine the appropriate sample delay value. Once TL0 behaviour is correct, checking function should be disabled to increase simulation speed.

### Explicit timings constructor

This constructor can be used to explicitly specify sampling times, when default values do not match the timing requirements of the slave.

OCP_TL0_TL1_Slave_Adapter(sc_module_name name,

, int ID

, sc_time CmdAcceptSampleDelay

, sc_time ResponseSampleDelay

, bool Check_setup_time = true

)

- name: Name of the module (slave adapter) instance.

- ID: Unique number identifying the Slave.

- CmdAcceptSampleDelay: Time number specifying the delay between the clock rising edge and the actual sampling time for 'SCmdAccept' signal.

- ResponseSampleDelay: Time number specifying the delay between the clock rising edge and the actual sampling time for response group signals.

- Check_setup_time: boolean value specifying if 'check_setup_time()’ function is executed during simulation (see below).

The smallest value for the sample delays is sc_get_time_resolution() ;it should be used in the case when TL0 master has pure 'zero-delay' signals. Users should always use a timed sample delay (and not a defined number of delta-cycles) since it is not possible to predict the number of delta-cycles required for signal stability.

## Basic TL0 Slave adapter with Datahandshake

The code for this adapter can be found in files layer_adapters/tl0_tl1/ocp_tl0_tl1_slave_adapter_hs(.h,cpp). During an OCP transaction, the adapter should behave as follows:

- For the request phase, the adapter catches each sent request on the the TL1 Slave Port, extracts the TL1 data fields and drives the corresponding TL0 Request group signals. Then, it waits for ‘SCmdAccept’ to be asserted by the slave and releases the request on TL1 channel.

- For the datahandshake phase, the adapter catches each sent data on the the TL1 Slave Port, extracts the TL1 data fields and drives the corresponding TL0 Data group signals. Then, it waits for ‘SDataAccept’ to be asserted by the slave and releases the data on TL1 channel.

- For the response phase, the adapter samples Response group signals driven by the TL0 Slave, test the ‘SResp’ signal to identify a valid response and conditionally put it on the TL1 channel. Then, it waits for the TL1 Master to release the response and asserts the ‘MrespAccept’ signal during one cycle.

Figure 3 TL0 Slave Adapter with Datahandshake

The adapter has to deal with potential late changes and glitches on its TL0 input signals. For a TL0 Slave, it concerns the following signals:

- SCmdAccept

- SDataAccept

- SResp and SData signals

The slave adapter issues some TL1 calls that can potentially change the state of the channel:

- Srelease() calls to release the request sent by the TL1 system

- SreleaseData() calls to release the data sent by the TL1 system

- SputResponse() calls to send response to the TL1 system

Once one of these calls has be done, the state channel has changed and there is no way to return to the previous state. Hence, it is essential to be sure that the TL0 signal values driving the TL1 calls are the good ones, i.e. they will not change until the end of the cycle.

The 'SputResponse()' and associated data class calls must happen only when TL0 associated signals 'SResp' and 'SData' are stable. Since the propagation delays for these signals are slave dependent, a 'Response Sample Delay' parameter is used to determine the precise instant when these signals should be sampled during the cycle (cf. Figure 2). Once sampled, 'SResp' is tested; if the response is valid (DVA value), dataclass response and 'SputResponse()' calls are done.

In the same manner; the 'Srelease()' call must happen only when 'SCmdAccept' is stable. Similarly, ‘SreleaseData()’ call must wait ‘SDataAccept’ to be stable. Since the propagation delay for these signals are slave dependent, a single ‘AcceptSampleDelay' parameter is used to determine when both 'SCmdAccept' AND ‘SDataAccept’ should be sampled during the cycle (cf Figure 4).

Note that we use a single sample delay for both signals for the following reasons:

- There is no combinational path between ‘SCmdAccept’ and ‘SDataAccept’, i.e there is only one signal to be sampled at each cycle.

- The OCP combinational timings guidelines (see below 2.3.3) specify that the output delay constraints are identical for both signals.

If output delays are different for each signal, ‘AcceptSampleDelay’ should be equal to the greatest value (see example 4, 2.3.4).

Figure 4 Slave Adapter sample delays (datahandshake)

### Default constructor

This constructor give defaults values to sampling delays. These values are compatible with the OCP Level 2 timing guidelines.

OCP_TL0_TL1_Slave_Adapter_HS(sc_module_name name,

, int ID

, bool Combinational

, sc_time ClockCycle

, bool Check_setup_time = true

)

- name: Name of the module (slave adapter) instance.

- ID: Unique number identifying the Slave.

- Combinational: Boolean specifying if there is a combinational path between DataHandshake and Response signals. Sampling times will be set as follows:

Combitional:

* AcceptSampleDelay = ClockCycle*99/100

* ResponseSampleDelay = ClockCycle*60/100

- Sequential:

* AcceptSampleDelay = ClockCycle*99/100

* ResponseSampleDelay = ClockCycle*60/100

- ClockCycle: sc_time number specifying the OCP clock period

- Check_setup_time: boolean value specifying if 'check_setup_time()’ function is executed during simulation (see below).

'Check_setup_time()' function is proposed for debugging purposes, to ensure that TL0 slave output signals do not change after the sample delays, which would cause the adapter to fail. A good way to proceed is to perform the first simulation with checking function activated to determine the appropriate sample delay value. Once TL0 behaviour is correct, checking function should be disabled to increase simulation speed.

### Explicit timings constructor

This constructor can be used to explicitly specify sampling times, when default values do not match the timing requirements of the slave.

OCP_TL0_TL1_Slave_Adapter_HS(sc_module_name name,

, int ID

, sc_time AcceptSampleDelay

, sc_time ResponseSampleDelay

, bool Check_setup_time = true

)

- name: Name of the module (slave adapter) instance.

- ID: Unique number identifying the Slave.

- AcceptSampleDelay: Time number specifying the delay between the clock rising edge and the actual sampling time for 'SCmdAccept' and ‘SDataAccept’ signals.

- ResponseSampleDelay: Time number specifying the delay between the clock rising edge and the actual sampling time for response group signals.

- Check_setup_time: boolean value specifying if 'check_setup_time()’ function is executed during simulation (see below).

## Examples

The following four examples (three for the basic adapter and one for the handshake adapter) present the use of the slave adapters in different system environments:

- Fully combinational transaction (Example 1).

- Synchronous split transaction with combinational ‘Accept’ signals (Example 2).

- Synchronous split transaction with synchronous ‘Accept’ signals (Example 3).

- Synchronous split transaction with combinational ‘Accept’ signals and datahandshake management (Example 4).

For each example, we discuss about the correct values to give to the ‘SampleDelay’ parameters in order to match the TL0 slave output delays.

### Fully combinational transaction (Example 1)

In this example, the Basic adapter connects a TL0 asynchronous slave to a TL1 asynchronous master. The top level C++ file is called top_slave_async.cpp. The example master and slave files are ocp_tl0_slave_async.cpp, ocp_tl1_master_async.cpp. Use Make_tl0_slave_async.gcc to build the example, and run ‘run_slave_async’ executable. Waveforms for OCP TL0 signals can be viewed using the generated VCD file ‘top_slave_async.vcd’.

This example presents a fully combinational OCP connection, where requests and responses occur in the same clock cycle. The master issues 5 writes followed by 5 reads, and checks received data, assuming that the OCP Target acts as a RAM. Operation timings are described in the header of the top_slave_async.cpp file.

Setting ‘CmdAcceptSampleDelay’ parameter:

The ‘SCmdAccept’ driving path is the following:

Clk (t=0)  ->  TL1_Master_MputRequest() (t=T1)   ->   TL0 ScmdAccept (t=T1+T2)

where :

- T1 specifies the delay (starting from the rising edge of the clock) used by the TL1 Master before issuing a request to the TL1 channel.

- T2 specifies the internal combinational delay of the TL0 slave between Request group input signals and the ‘SCmdAccept’ signal. This feature is slave-dependent

For this kind of combinational path on ‘SCmdAccept’, parameter ‘CmdAcceptSampleDelay’ must be greater than T1 + T2.

Setting ‘ResponseSampleDelay’ parameter:

The response group signals driving path is the following:

Clk (t=0)  ->  TL1_Master_MputRequest() (t=T1)   ->   TL0 Response group (t=T1+T3)

where :

- T1 specifies the delay (starting from the rising edge of the clock) used by the TL1 Master before issuing a request to the TL1 channel.

- T3 specifies the internal combinational delay of the TL0 slave between Request group input signals and Response group output signals. This feature is slave-dependent.

For this kind of combinational path on response signals, parameter ‘ResponseSampleDelay’ must be greater than T1 +T3.

### Synchronous split transaction with combinational ‘Accept’ signals (Example 2)

In this example, the Basic adapter connects a TL0 synchronous slave to a TL1 master. The top level C++ file is called top_slave_sync.cpp. The example master and slave files are ocp_tl0_slave_sync.cpp, ocp_tl1_master_async.cpp. Use Make_tl0_slave_sync.gcc to build the example, and run ‘run_slave_sync’ executable. Waveforms for OCP TL0 signals can be viewed using the generated VCD file ‘top_slave_sync.vcd’.

In this example, responses occurs one cycle after requests, and 'Accept' signals use combinational paths. The master issues 5 writes followed by 5 reads, and checks received data , assuming that the OCP Target acts as a RAM. Operation timings are described in the header of the top_slave_sync.cpp file.

Setting ‘CmdAcceptSampleDelay’ parameter:

The value is the same as for example 1: ‘CmdAcceptSampleDelay’  must be greater than T1 + T2

Setting ‘ResponseSampleDelay’ parameter:

The response group signals driving path is the following:

Clk (t=0)  ->   Response group (t=T3)

where :

- T3 specifies the internal combinational delay of the TL0 slave between the rising edge of the clock and the response group assertion time. This feature is slave-dependent.

For this kind of combinational path on response signals, parameter ‘ResponseSampleDelay’ must be greater than T3.

NOTE: OCP combinational timings guidelines

In this example, the TL1 master has its 'MCmd' propagation delays equal to zero, and uses a split, synchronous request/response scheme. As a result, delays seen by the adapter on Response signals and 'SCmdAccept' are equal to the real output delays of the TL0 Slave. Hence, it becomes possible to check the compliance of the TL0 Slave to the OCP timings guideline (specified in the OCP Specification Release 1.0, Chapter 16 & 17).

By setting the sample delay values according to the Level-1 and Level-2 timing guidelines, we can verify that the TL0 Slave is only Level-2 compliant.

More generally, it is possible to check the compliance of a TL0 Slave/Master when the following conditions are fulfilled:

- TL1 system connected to the TL1 channel issues interface channel calls only at clock rising edges. In practical terms, the TL1 system must only use clocked process to issue TL1 calls and can not include any ‘wait()’ calls in the calling processes. In this way, the TL1 delays resulting from ‘wait()’ statements do not have an impact on final TL0 output delays.

- There is no combinational path between the Master/Slave and the corresponding adapter. In this way, the ‘Sample Delays’ of the adapters do not have an impact on final TL0 output delays.

### Synchronous split transaction with synchronous ‘Accept’ signals (Example 3)

In this example, the Basic adapter connects a TL0 synchronous slave to a TL1 master. The top level C++ file is called top_slave_sync2.cpp. The example master and slave files are ocp_tl0_slave_sync2.cpp, ocp_tl1_master_async.cpp. Use Make_tl0_slave_sync2.gcc to build the example, and run ‘run_slave_sync2’ executable. Waveforms for OCP TL0 signals can be viewed using the generated VCD file ‘top_slave_sync2.vcd’.

In this example, responses occurs one cycle after requests, and 'Accept' signals assertion take one cycle ( + some propagation delays). The master issues 5 writes followed by 5 reads, and checks received data , assuming that the OCP Target acts as a RAM. Operation timings are described in the header of the top_slave_sync2.cpp file.

Setting ‘CmdAcceptSampleDelay’ parameter:

The ‘SCmdAccept’ driving path is the following:

Clk (t=0)  ->   TL0 ScmdAccept (t=T2)

where :

- T2 specifies the internal combinational delay of the TL0 slave between the rising edge of the clock and the ‘SCmdAccept’ assertion time. This feature is slave-dependent.

For this kind of combinational path on response signals, parameter ‘ResponseSampleDelay’ must be greater than T2.

Setting ‘ResponseSampleDelay’ parameter:

The value is the same as for example 2: ‘ResponseSampleDelay’ greater than T3

### Synchronous split transaction with combinational ‘Accept’ signals and datahandshake management (Example 4)

In this example, we use the second slave adapter (Basic signals + datahandshake) to connect a TL0 synchronous slave with datahandshake support to a TL1 master. The top level C++ file is called top_slave_sync_hs.cpp. The example master and slave files are ocp_tl0_slave_sync_hs.cpp, ocp_tl1_master_async_hs.cpp. Use Make_tl0_slave_sync_hs.gcc to build the example, and run ‘run_slave_sync_hs’ executable. Waveforms for OCP TL0 signals can be viewed using the generated VCD file ‘top_slave_sync_hs.vcd’.

In this example, responses occurs one cycle after requests, data are transmitted one cycle after corresponding requests (datahandshake), and both 'SCmdAccept' and ‘SdataAccept’ signals use combinational paths. The master issues 5 writes followed by 5 reads, and checks received data , assuming that the OCP Target acts as a RAM. Operation timings are described in the header of the top_slave_sync_hs.cpp file.

Setting ‘AcceptSampleDelay’ parameter:

The ‘SCmdAccept’ driving path is the following:

Clk (t=0)  ->  TL1_Master_MputRequest() (t=T1)   ->   TL0 ScmdAccept (t=T1+T2)

where :

- T1 specifies the delay (starting from the rising edge of the clock) used by the TL1 Master before issuing a request to the TL1 channel.

- T2 specifies the internal combinational delay of the TL0 slave between Request group input signals and the ‘SCmdAccept’ signal. This feature is slave-dependent

The ‘SDataAccept’ driving path is the following:

Clk (t=0)  ->  TL1_Master_MputDataRequest() (t=T3)   ->   TL0 ScmdAccept (t=T3+T4)

where :

- T3 specifies the delay (starting from the rising edge of the clock) used by the TL1 Master before issuing a data request to the TL1 channel.

- T4 specifies the internal combinational delay of the TL0 slave between Data Request group input signals and the ‘SDataAccept’ signal. This feature is slave-dependent

Since the same delay is used to sample both ‘SCmdAccept’ and ‘SdataAccept’ signals,  the ‘AcceptSampleDealy’ has to be set to the maximum of these two values: max(T1+T2, T3+T4).

Setting ‘ResponseSampleDelay’ parameter:

The value is the same as for example 2: ‘ResponseSampleDelay’ greater than T5’

where :

- T5 specifies the internal combinational delay of the TL0 slave between the rising edge of the clock and the response group assertion time. This feature is slave-dependent.

# TL0 Master Adapter

These adapters enable to connect a TL0 Master to a TL1 system. In this section, we will present two examples of adapters that implement two different subsets of the OCP interface:

- Basic adapter: All the Basic signals (Clock, MAddr, MCmd, Mdata, ScmdAccept, Sdata, Sresp) + MRespAccept

- Basic adapter with datahandshake: All the Basic signals + MRespAccept + Datahandshake signals (MdataVaild + SDataAccept)

Four examples are provided (three for the basic adapter and one for the datahandshake adapter), which present the use of the master adapters with different kinds of TL0 masters and TL1 slaves.

## Basic TL0 Master adapter

The code for this adapter can be found in files layer_adapters/tl0_tl1/ocp_tl0_tl1_master_adapter(.h,cpp). During an OCP transaction, the adapter should behave as follows:

- For the request phase, the adapter samples all the TL0 request group signals driven by the TL0 master, test the ‘MCmd’ signal to identify a valid request and conditionally put a request on the TL1 channel. Then, it waits for the TL1 Slave to release the request and asserts the ‘SCmdAccept’ signal during one cycle.

- For the response phase, the adapter catches each sent response on its TL1 Master Port, extracts the TL1 data fields and drives the corresponding TL0 response group signals. Then, it waits for ‘MRespAccept’ to be asserted by the master and releases the response on TL1 channel.

Figure 5 TL0 Master Adapter

The adapter has to deal with potential late changes and glitches on its TL0 input signals. For a TL0 master, it concerns the following signals:

- MCmd, MData and MAddr

- MRespAccept

The master adapter issues some TL1 calls that can potentially change the state of the channel:

- Mput*Request() calls to send a request to the TL1 system

- Mrelease() calls to release responses sent by the TL1 system

Once one of these calls has been done, the state channel has changed and there is no way to return to the previous state. Hence, it is essential to be sure that the TL0 signal values driving the TL1 calls are the good ones, i.e. they will not change until the end of the cycle.

The 'Mput*Request()' and associated data class calls must only happen when TL0 associated signals 'MCmd', 'MData' and 'MAddr' are stable. Since the propagation delays for these signals are master dependent, a 'RequestSampleDelay' parameter is used to determine when these signals should be sampled during the cycle. Once sampled, 'MCmd' is tested and corresponding 'Mput*Request' call is issued to the channel.

The 'Mrelease()' call must happen only when 'MRespAccept' is stable. Since the propagation delay for this signal is master dependent, a 'RespAcceptSampleDelay' parameter is used to determine when 'MRespAccept' should be sampled during the cycle (cf. Figure 6).

Figure 6 Master Adapter sample delays

### Default constructor

This constructor give defaults values to sampling delays. These values are compatible with the OCP Level 2 timing guidelines.

OCP_TL0_TL1_Master_Adapter(sc_module_name name,

, int ID

, int Priority

, bool Combinational

, sc_time ClockCycle

, bool Check_setup_time = true

)

- name: Name of the module (slave adapter) instance.

- ID: Unique number identifying the Master.

- Priority: Positive number specifying the priority relative to the other masters for bus access. Higher numbers means higher priority. Masters can have the same priority.

- Combinational: Boolean specifying if there is a combinational path between Request and Response signals. Sampling times will be set as follows:

Combitional:

* RequestSampleDelay = ClockCycle*60/100

* ResponseSampleDelay = ClockCycle*75/100

- Sequential:

* RequestSampleDelay = ClockCycle*60/100

* ResponseSampleDelay = ClockCycle*75/100

- ClockCycle: sc_time number specifying the OCP clock period

- Check_setup_time: boolean value specifying if 'check_setup_time()’ function is executed during simulation (see below).

'Check_setup_time()' function is proposed for debugging purposes, to ensure that TL0 master output signals don't change after the sample delays, which would cause the adapter to fail. A good way to proceed is to perform the first simulation with checking function activated to determine the appropriate sample delay value. Once TL0 behaviour is correct, checking function should be disabled to increase simulation speed.

### Explicit timings constructor

This constructor can be used to explicitly specify sampling times, when default values do not match the timing requirements of the master.

OCP_TL0_TL1_Master_Adapter(sc_module_name name,

, int ID

, int Priority

, sc_time RequestSampleDelay

, sc_time RespAcceptSampleDelay

, bool Check_setup_time = true

)

- name: Name of the module (slave adapter) instance.

- ID: Unique number identifying the Master.

- Priority: Positive number specifying the priority relative to the other masters for bus access. Higher numbers means higher priority. Masters can have the same priority.

- RequestSampleDelay: Time number specifying the delay between the clock rising edge and the actual sampling time for TL0 request group signals.

- RespAcceptSampleDelay: Time number specifying the delay between the clock rising edge and the actual sampling time for ‘MrespAccept’ signal.

- Check_setup_time: boolean value specifying if 'check_setup_time()’ function is executed during simulation (see below).

The smallest value for the sample delays is sc_get_time_resolution() ; it should be used in the case when TL0 master has pure 'zero-delay' signals. Users should always use a timed sample delay (and not a defined number of delta-cycles) since it is not possible to predict the number of delta-cycles required for signal stability.

## Basic TL0 Master adapter with Datahandshake

The code for this adapter can be found in files layer_adapters/tl0_tl1/ocp_tl0_tl1_master_adapter_hs(.h,cpp). During an OCP transaction, the adapter should behave as follows:

- For the request phase, the adapter samples Request group signals driven by the TL0 master, tests the ‘MCmd’ signal to identify a valid request and conditionally puts it on the TL1 channel. Then, it waits for the TL1 Slave to release the request and asserts the ‘SCmdAccept’ signal during one cycle.

- For the datahandshake phase, the adapter samples Data Request group signals driven by the TL0 master, tests the ‘MDataValid’ signal to identify a valid data request and conditionally puts it on the TL1 channel. Then, it waits for the TL1 Slave to release the data request and asserts the ‘SDataAccept’ signal during one cycle.

- For the response phase, the adapter catches each sent request on its TL1 Master Port, extracts the TL1 data fields and drives the corresponding TL0 Response group signals. Then, it waits for ‘MRespAccept’ to be asserted by the master and releases the response on TL1 channel.

Figure 7 TL0 Master Adapter with Datahandshake

The adapter has to deal with potential late changes and glitches on its TL0 input signals. For a TL0 Master, it concerns the following signals:

- MCmd, Mdata

- MDataValid, MData

- MRespAccept

The master adapter issues some TL1 calls that can potentially change the state of the channel:

- Mput*Request() calls to send a request to the TL1 system

- MputDataRequest() calls to send data to the TL1 system

- Mrelease() calls to release responses sent by the TL1 system

Once one of these calls has been done, the state channel has changed and there is no way to return to the previous state. Hence, it is essential to be sure that the TL0 signal values driving the TL1 calls are the good ones, i.e. they will not change until the end of the cycle.

The 'Mput*Request()' and associated data class calls must happen only when TL0 associated signals 'MCmd' and 'MAddr' are stable. Similarly, 'MputDataRequest()’ call must wait ‘MDataValid’ and ‘MData’ to be stable. Since the propagation delays for these signals are master dependent, we use a 'RequestSampleDelay' parameter to determine when these signals should be sampled during the cycle.

In the same way; the 'Mrelease()' call must happen only when 'MRespAccept' is stable. Since the propagation delay for this signal is master dependent, a 'RespAcceptSampleDelay' parameter is used to determine when 'MRespAccept' should be sampled during the cycle (cf Figure 8).

Note that we use a single sample delay for request and datahandshake for the following reasons:

- There is no combinational path between request group and datahandshake group signals, i.e there is only one group to be sampled at each cycle.

- The OCP combinational timings guidelines (see 2.3.3) specifies that the output delay constraints are identical for both groups.

In the case where output delays are different for each group, ‘AcceptSampleDelay’ should be equal to the greatest value (see example 4, 3.3.4).

Figure 8 Master Adapter sample delays (datahandshake)

### Default constructor

This constructor give defaults values to sampling delays. These values are compatible with the OCP Level 2 timing guidelines.

OCP_TL0_TL1_Master_Adapter_HS(sc_module_name name,

, int ID

, int Priority

, bool Combinational

, sc_time ClockCycle

, bool Check_setup_time = true

)

- name: Name of the module (slave adapter) instance.

- ID: Unique number identifying the Master.

- Priority: Positive number specifying the priority relative to the other masters for bus access. Higher numbers means higher priority. Masters can have the same priority.

- Combinational: Boolean specifying if there is a combinational path between DataHandshake and Response signals. Sampling times will be set as follows:

Combitional:

* RequestSampleDelay = ClockCycle*60/100

* RespAcceptSampleDelay = ClockCycle*75/100

- Sequential:

* RequestSampleDelay = ClockCycle*60/100

* RespAcceptSampleDelay = ClockCycle*75/100

- ClockCycle: sc_time number specifying the OCP clock period

- Check_setup_time: boolean value specifying if 'check_setup_time()’ function is executed during simulation (see below).

'Check_setup_time()' function is proposed for debugging purposes, to ensure that TL0 master output signals don't change after the sample delays, which would cause the adapter to fail. A good way to proceed is to perform the first simulation with checking function activated to determine the appropriate sample delay value. Once TL0 behaviour is correct, checking function should be disabled to increase simulation speed.

### Explicit timings constructor

This constructor can be used to explicitly specify sampling times, when default values do not match the timing requirements of the master.

OCP_TL0_TL1_Master_Adapter_HS(sc_module_name name,

, int ID

, int Priority

, sc_time RequestSampleDelay

, sc_time RespAcceptSampleDelay

, bool Check_setup_time = true

)

- name: Name of the module (slave adapter) instance.

- ID: Unique number identifying the Master.

- Priority: Positive number specifying the priority relative to the other masters for bus access. Higher numbers means higher priority. Masters can have the same priority.

- RequestSampleDelay: Time number specifying the delay between the clock rising edge and the actual sampling time for TL0 request AND datahandshake group signals.

- RespAcceptSampleDelay: Time number specifying the delay between the clock rising edge and the actual sampling time for ‘MrespAccept’ signal.

- Check_setup_time: boolean value specifying if 'check_setup_time()’ function is executed during simulation (see below).

The smallest value for the sample delays is sc_get_time_resolution() ; it should be used in the case when TL0 master has pure 'zero-delay' signals. Users should always use a timed sample delay (and not a defined number of delta-cycles) since it is not possible to predict the number of delta-cycles required for signal stability.

## Examples

The following four examples (three for the basic adapter and one for the handshake adapter) present the use of the master adapters in different system environments:

- Fully combinational transaction (Example 1).

- Synchronous split transaction with combinational ‘Accept’ signals (Example 2).

- Synchronous split transaction with synchronous ‘Accept’ signals (Example 3).

- Synchronous split transaction with combinational ‘Accept’ signals and datahandshake management (Example 4).

For each example, we discuss about the correct values to give to the ‘SampleDelay’ parameters in order to match the TL0 master output delays.

### Fully combinational transaction (Example 1)

In this example, the Basic adapter connects a TL0 asynchronous master to a TL1 asynchronous slave. The top level C++ file is called top_master_async.cpp. The example master and slave files are ocp_tl0_master_async.cpp, ocp_tl1_slave_async.cpp. Use Make_tl0_master_async.gcc to build the example, and run ‘run_master_async’ executable. Waveforms for OCP TL0 signals can be viewed using the generated VCD file ‘top_master_async.vcd’.

This example presents a fully combinational OCP connection, where requests and responses occur in the same clock cycle. The master issues 5 writes followed by 5 reads, and checks received data. Operation timings are described in the header of the top_master_async.cpp file.

Setting ‘RequestSampleDelay’ parameter:

The driving path for the request group signals is the following:

Clk (t=0)  ->  TL0 Request Group (t=T1)

where :

- T1 specifies the delay (starting from the rising edge of the clock) used by the TL0 Master before issuing a request.

For this kind of combinational path on request group signals, parameter ‘RequestSampleDelay’ must be greater than T1.

Setting ‘RespAcceptSampleDelay’ parameter:

The driving path for the response group signals is the following:

Clk (t=0)  ->  TL0 Request Group (t=T1)   ->   TL0 Adapter Request sampling (t=T5)   ->   TL1 Slave Response (t=T5+T3)   ->   TL0 Master ‘MrespAccept’ (t=T5+T3+T4)

where :

- T1 specifies the delay (starting from the rising edge of the clock) used by the TL0 Master before issuing a request.

- T5 specifies the ‘RequestSampleDelay’ parameter.

- T3 specifies the delay (starting from the request reception) used by the TL1 Slave before issuing a response.

- T4 specifies the delay (starting from the response reception) used by the TL0 Master before issuing a response acknowledge.

For this kind of combinational path on ‘MRespAccept’ response signals, parameter ‘MRespAcceptSampleDelay’ must be greater than ‘RequestSampleDelay’ + T4+T3.

### Synchronous split transaction with combinational ‘Accept’ signals (Example 2)

In this example, the Basic adapter connects a TL0 synchronous master to a TL1 slave. The top level C++ file is called top_master_sync.cpp. The example master and slave files are ocp_tl0_master_async.cpp, ocp_tl1_slave_sync.cpp. Use Make_tl0_master_sync.gcc to build the example, and run ‘run_master_sync’ executable. Waveforms for OCP TL0 signals can be viewed using the generated VCD file ‘top_master_sync.vcd’.

In this example, responses occurs one cycle after requests, and 'Accept' signals use combinational paths. The master issues 5 writes followed by 5 reads, and checks received data. Operation timings are described in the header of the top_master_sync.cpp file.

Setting ‘RequestSampleDelay’ parameter:

The value is the same as for example 1: ‘RequestSampleDelay’  must be greater than T1

Setting ‘RespAcceptSampleDelay’ parameter:

The driving path for the response group signals is the following:

Clk (t=0)  ->  TL1 Slave Response (t=T3)   ->   TL0 Master ‘MRespAccept’ (t=T3+T4)

where :

- T3 specifies the delay (starting from the rising edge of the clock following a request reception) used by the TL1 Slave before issuing a response.

- T4 specifies the delay (starting from the response reception) used by the TL0 Master before issuing a response acknowledge.

For this kind of combinational path on ‘MRespAccept’ response signals, parameter ‘MRespAcceptSampleDelay’ must be greater than T3+T4.

### Synchronous split transaction with synchronous ‘Accept’ signals (Example 3)

In this example, the Basic adapter connects a TL0 synchronous master to a TL1 slave. The top level C++ file is called top_master_sync2.cpp. The example master and slave files are ocp_tl0_master_async.cpp, ocp_tl1_slave_sync.cpp. Use Make_tl0_master_sync2.gcc to build the example, and run ‘run_master_sync2’ executable. Waveforms for OCP TL0 signals can be viewed using the generated VCD file ‘top_master_sync2.vcd’.

In this example, responses occur one cycle after requests, and 'Accept' signals assertion take one cycle ( + some propagation delays). The master issues 5 writes followed by 5 reads, and checks received data. Operation timings are described in the header of the top_master_sync2.cpp file.

Sample delays for this example are the same as for example 2.

### Synchronous split transaction with combinational ‘Accept’ signals and datahandshake management (Example 4)

In this example, we use the second slave adapter (Basic signals + datahandshake) to connect a TL0 synchronous master with datahandshake support to a TL1 slave. The top level C++ file is called top_master_sync_hs.cpp. The example master and slave files are ocp_tl0_master_sync_hs.cpp, ocp_tl1_slave_sync_hs.cpp. Use Make_tl0_master_sync_hs.gcc to build the example, and run ‘run_master_sync_hs’ executable. Waveforms for OCP TL0 signals can be viewed using the generated VCD file ‘top_master_sync_hs.vcd’.

In this example, responses occurs one cycle after requests, data are transmitted one cycle after corresponding requests (datahandshake), and both 'SCmdAccept' and ‘SDataAccept’ signals use combinational paths. The master issues 5 writes followed by 5 reads, and checks received data. Operation timings are described in the header of the top_master_sync_hs.cpp file.

Setting ‘RequestSampleDelay’ parameter:

The driving path for the request group signals is the following:

Clk (t=0)  ->  TL0 Request Group (t=T1)

where :

- T1 specifies the delay (starting from the rising edge of the clock) used by the TL0 Master before issuing a request.

The driving path for the datahandshake group signals is the following:

Clk (t=0)  ->  TL0 Request Group (t=T2)

where :

- T3 specifies the delay (starting from the rising edge of the clock) used by the TL0 Master before issuing a data.

Since the same delay is used to sample both Request and Datahandshake signals, the ‘RequestSampleDealy’ has to be set to the maximum of these two values: max(T1, T2).

Setting ‘RespAcceptSampleDelay’ parameter:

The value is the same as for example 2.

# OCP Extension GUIDELINES

The presented examples use a subset of the OCP 1.0 dataflow interface, including :

- Basic signals

- ‘MRespAccept’ signal

- Datahandshake signals

Extension of the adapters to support the full OCP 1.0 dataflow interface require somes changes to the current adapters. We give here some guidelines to implement these extensions.

## Additional Request group signals

The following request group signals are missing: MAddrSpace, MBurst, MbyteEn, MConnID and MthreadID. Since these signals belong to the same group as the basic signals, their implementation is straightforward and just require to:

- Add the corresponding TL0 signals on the adapter interface

- For the Master adapter, add the corresponding Request Group Master TL1 calls (MputM*() ) at the same place in the code than those used for basic signals.

- For the Slave adapter, add the corresponding Request Group TL0 signal write calls at the same place in the code than those used for basic signals.

## Additional Datahandshake group signals

The only missing signal is ‘MDataThreadID’. As for request group signals, extension is simple and just require to:

- Add the TL0 signal on the adapter interface.

- For the Master adapter, add a MputMDataThreadID() at the same place as MputMDataHS() call.

- For the Slave adapter, add a SgetMDataThreadID()at the same place as SgetMDataHS() call

## Threadbusy signals

In the OCP 1.0 release, Threadbusy signals are just a hint for the receiving agents. It means that there is no specific time for the receiving agent to take into account the threadbusy signals.

The easiest way to implement threadbusy in the adapters is just to add an SC_METHOD which is sensitive on the rising edge of the clock, tests the TL0 threadbusy signal and issue the corresponding TL1 call to the channel. This way, there is no need for an additional sample delay, but it is no possible to model exact threadbusy behaviours as described in the future release OCP 2.0, due to the extra latency cycle introduced by the SC_METHOD.

To model the exact threadbusy behaviour, the only solution is to add a third sample delay triggering the threadbusy reading process.